PART 1--GENERAL  (Draperies Ripplefold)

- DESCRIPTION

  - General: Provide operable draperies in accordance with this document and plans.

  - Related work specified elsewhere.

    - Rough carpentry for wood blocking and support grounds.

    - Miscellaneous metal for framing and reinforcing.

    - Optional: Electrical power requirements and controls for motorized draperies.

- QUALITY ASSURANCE

  - Provide operable draperies from a firm that has specialized in the fabrication and installation of such work for at least five (5) years in projects of similar size and scope.

  - Provide operable draperies as complete units produced by a single manufacturer, including the necessary mounting brackets, hardware, fittings, fastenings, and installation.

  - Installation shall be performed by an authorized manufacturer's representative experienced in the installation and maintenance of such assemblies.

- STANDARDS

  - Except as modified by governing codes and contract documents, comply with applicable provisions and recommendations of the following:

    - when required by fire codes, fabrics shall be flame resistant in accordance with NFPA-701 test requirements.

    - Flameproofing shall leave the fabric cleanable, dimensionally stable, and resistant to UV deterioration.

- SUBMITTALS

  - Manufacturer's Data: Submit specifications and operating and installation instructions for draperies.

  - Shop Drawings: Submit drawings for draperies, including dimensioned plans seaming instructions, details, connections, anchorages, and the relation to adjacent construction.

  - Samples: Submit 24 x 24-in. samples of fabrics(s) as specified and operable hardware with cut sheets.

- PRODUCT DELIVERY, STORAGE AND HANDLING

  - Subcontractor shall not deliver the draperies until room(s) containing draperies are painted and ready for drapery installation. Draperies and components shall be protected from damage during delivery and installation.

- WARRANTY

  - A written warranty agreeing to repair or replace hardware or fabric due to faulty material, workmanship or installation for a period of one (1) year.

PART 2--Product: MANUFACTURER

  - Acceptable Manufacturer: Hunter Douglas Hospitality Window Coverings – 9900 Gidley St, El Monte, CA  91731

  - General: Provided an operable drapery system as produced by one manufacturer. Products specified herein by proprietary designation are as manufactured by HD Hospitality, at 800-229-5300.

  - Equivalent products of other manufacturers will be considered provided they meet those established standards and comply with the contract documents.

- Ripplefold Drapery

  - Ripplefold Drapery uses a permanently stiffened nylon snap-tape that supports the header.

  - Drapery fullness shall be [60%, 80%, 100% or 120%]

  - Side hems shall be double turned 1 1/2 in. wide.

  - Bottom hems shall to be 4 in. double turned

  - Drapery Hardware: Available in [cord-drawn, baton drawn, hand-drawn or motorized] track options using [ceiling mounted, wall mounted] hardware.

  - Drapery Track Finish: [Standard straight track, Optional Curved track with a radius of 12”] in color [Standard White, or Optional Black]

  - Track Operation: [Inoperable, One-way stack Right-hand drawn, One-way stack Left-hand drawn, Two-way stack split drawn with overlapping center carrier, Two-way stack split drawn with butt carrier]

  - Drapery Lining: [Non-blackout lining, 2-pass blackout lining, 3-pass blackout lining, or unlined]

  - Size: use drawing to scale to approx. size. All finished dimensions to be verified in field.

- FABRICATION

      - Lined Ripplefold drapery header shall be fabricated as follows: 1 inch permanently stiffened nylon snap-tape sewn onto a 1-½ inch double-folded header using a double-needle finish and sewn with mercerized cotton thread.

      - Side hems: 1-½" double turn fabric consists of three layers and blind stitched so that stitches are not visible on the face of the fabric. All vertical seaming shall be sewn with a twin-needle sewing machine with salvaged trimmed and overlocked. All vertical seams shall lie flat and straight without puckering.

      - Vertical Seams: Where drapery widths are sewn together in vertical seams, patterned fabrics widths shall be joined so that all patterns match perfectly. All vertical seaming shall be sewn with a twin-needle sewing machine with salvaged trimmed and overlocked. All vertical seams shall lie flat and straight without puckering.

      - Bottom Seams: 4” double turn hems to consist of three layers of fabric, and blind-stitched. [Standard 1-in.-sq. lead weights shall be securely sewn at each seam and at the corners, or Optional string weights shall be inserted in hems for sheer or light fabrics]

      - Drapery fullness: The finished drapery width will be calculated to meet the fullness as driven by the carrier count in the finished drapery track width. If less than a full width of fabric is required to accomplish the specified fullness, no less than one-half width shall be used, and they shall be located at the extreme ends of each drapery panel.

      - Lining: widths shall be equal to the width of the face fabric, joined to the face fabric at the top using an inside-invisible seam. The side hem will be a 1 1/2-in. return of the face fabric

- FABRIC(S)

  - Fabric and color to be chosen by specifier: To be fabric rated for use as drapery, avoid fabrics rated for use as upholstery.

PART 3--Execution

- EXAMINATION

  - Examine substrates, adjoining construction and conditions under which the work is to be installed. The work shall not proceed until unsatisfactory conditions have been corrected.

- PREPARATION

  - Install draperies with hardware in accordance with manufacturer's recommendations.

  - Hang draperies so that the top of the heading clears the ceiling above by 1/4 in. and the bottom hem clearance is a maximum of 1 in. from the finished floor or 1/2 in. from the sill line.

  - Draperies shall be free of creases and wrinkles after hanging and shall be steamed and creases dressed down as required.

- PROTECTION 
Protect draperies from soiling or damage after hanging.

END OF SECTION